SECTION 105500 -- Postal Specialties

SECTION 105513 – Central Mail Delivery Boxes

Cluster Mailbox Units

Display hidden notes to specifier. (Don’t know how? Click Here)

** NOTE TO SPECIFIER ** Florence Manufacturing Company; commercial and residential mailboxes and accessories.
 
 This section is based on the products of Florence Manufacturing Company, which is located at: 
  5935 Corporate Drive
  Manhattan, KS 66503

BudgetMailboxes.com is an authorized dealer of Florence products.
  Tel:  (866) 707-0008
  Fax:  (619) 324-5809
  Email:  customercare@weblifestores.com
  Web:  http://www.budgetmailboxes.com

Florence Manufacturing has focused on providing superior quality mailboxes and accessories for centralized distribution and collection of mail and packages since 1934. This laser-like focus means Florence has the most extensive industry knowledge and has grown its product lines to provide the best solutions for design professionals and postal officials throughout ever changing times.

The Florence line of postal specialties has maintained industry leadership for decades by offering consistently superior product quality and rapid, reliable customer service – even after the sale. Technical support and customer service are both provided through a centralized customer service operation; a dedicated group of engineers, quality manufacturers and support all stand ready to help your centralized mail delivery solution come to life.

Florence Manufacturing continues to dedicate itself to providing secure postal specialty products built to the latest USPS standards and is the only manufacturer authorized to supply “F” series cluster box units (CBU), outdoor parcel lockers (OPL) and replacement pedestals and parts directly to the USPS since being awarded their single-source contract for outdoor delivery equipment in 2005. With the most comprehensive USPS Approved commercial and residential mailbox product lines in the market, Florence Manufacturing is proud to be your partner in mailbox and storage solutions. 
 
 SECTION 105523 - MAILBOXES

- GENERAL

  - SECTION INCLUDES

** NOTE TO SPECIFIER ** Delete items below not required for project.

    - Front-loading exterior mailboxes.

- Cluster Box Unit (CBU)

- High-Security Cluster Box Unit (HSCBU)

- Outdoor Parcel Locker (OPL)

    - Decorative accessories.

- Traditional vogue

- Classic vogue

  - RELATED SECTIONS

** NOTE TO SPECIFIER ** Delete sections below not relevant to this project; add others as required.

    - Section 048000 - Masonry Assemblies.

    - Section 055000 - Metal Fabrications: Metal anchors.

    - Section 033000 – Cast-in-place concrete.

  - REFERENCES

- United States Postal Service (USPS).

1)	USPS-B-1118: United States Postal Service Standard, Cluster Box Units

2)	USPS Publication 16.

- ASTM A 591 - Specification for Steel Sheet, Electrolytic Zinc Coated, for Light Coating Mass Applications

- ASTM A 653 - Specification For Steel Sheet, Zinc Coated (Galvanized), or Zinc-Iron Alloy Coated (Galvannealed) by the Hot Dip Process.

- ASTM A 666 - Specification for Austenitic Stainless Steel Sheet, Strip, Plate and Flat Bar.

- ASTM A 1008 - Specification for Steel, Sheet Cold-Rolled, Carbon, Structural High Strength Low-Alloy and High Strength Low-Alloy with Improved Formability.

- ASTM B 209 - Specification Aluminum and Aluminum Alloy Sheet and Plate.

- ASTM B 221 - Specification Aluminum and Aluminum Alloy Extruded Bar, Rods, Wire, Shapes, and Tubes.

  - SUBMITTALS

    - Submit under provisions of Section 013000.

    - [Product Data]:  Provide manufacturer's standard catalog data for specified products including:

    - Construction details, material descriptions, dimensions and finishes.

    - Preparation instructions and recommendations.

    - Storage and handling instructions and recommendations.

    - Installation methods.

    - Care and cleaning instructions and recommendations.

- Shop Drawings:  Prepared specifically for this project; show dimensions of mailboxes, and interface with other products.

** NOTE TO SPECIFIER ** Delete selection samples if colors have already been selected.

- Selection Samples:	For each finish product specified, one complete set of color chips representing manufacturer’s full range of available colors and patterns.

- Verification Samples:	 For each finish product specified, one sample, minimum size two inches (51 mm) square, representing actual color and texture.

  - REGULATORY REQUIREMENTS

    - Comply with USPS-B-1118 Specification.

    - Comply with Americans with Disabilities Act Accessibility Guidelines (ADAAG).

  - QUALITY ASSURANCE

    - Manufacturer Qualifications:  Manufacturer shall have a Quality System in place to ensure and be able to substantiate that manufactured units conform to requirements and match the approved design and must be ISO 9001:2008 certified.

  - DELIVERY, STORAGE, AND HANDLING

- Inspect the materials upon delivery to assure that specified products have been received.

- Store materials protected from exposure to harmful weather conditions.

- Handle materials to prevent damage or marring of finish.

  - WARRANTY

    - Manufacturer's standard warranty to repair or replace components of postal specialties that fail in materials or workmanship within five (5) years from date of purchase.

- PRODUCTS

  - MANUFACTURERS

- Acceptable Manufacturer:  Florence Manufacturing, 5935 Corporate Drive, Manhattan, KS 66503. Tel: (800) 275-1747, Fax: (800) 275-5081. Email: sales@auth-florence.com. Web: www.florencemailboxes.com.

- Substitutions:   Not permitted.

- Refer to USPS Approved Product manufacturer list as published annually.

** NOTE TO SPECIFIER ** Delete entire 2.2 below if not required.

2.2 	CLUSTER BOX UNITS (CBU)

Cluster Box Units: Must be an officially licensed product of the USPS. Consisting of multiple compartments enclosed within freestanding, pedestal-mounted aluminum cabinet. Provide access to compartments for distributing incoming mail and collecting outgoing mail from front of unit by unlocking master lock and swinging pair of side-hinged master doors to provide accessibility to entire group of compartments. Provide access to each compartment for removing mail by swinging compartment door. Provide slot for outgoing mail collection with weather protection hood. Comply with USPS-B-1118 (“F” Specification) as manufactured by Florence Manufacturing.

** NOTE TO SPECIFIER ** Delete model type not required in project.

- Model:  vital™ series 1570 Cluster Box Unit (CBU), the Florence vital™ 1570 Cluster Box Unit is an officially licensed product of the USPS.

  - Model 1570-8, Type I: 8 compartments, 3.25 inches (83 mm) high, with 2 parcel compartments and 1 outgoing mail receptacle.

  - Model 1570-12, Type II: 12 compartments, 3.25 inches (83 mm) high, with 1 parcel compartment and 1 outgoing mail receptacle.

  - Model 1570-16, Type III: 16 compartments, 3.25 inches (83 mm) high, with 2 parcel compartments and 1 outgoing mail receptacle.

  - Model 1570-13, Type IV: 13 compartments, 5 inches (127 mm) high, with 1 parcel compartment and 1 outgoing mail receptacle.

  - Model 1570-4T5, Type V: 4 compartments, 6.5 inches (165 mm) high, with 2 parcel compartments and 1 outgoing mail receptacle.

  - Model 1570-8T6, Type VI: 8 compartments, 3.25 inches (83 mm) high, with 4 parcel compartments and 1 outgoing mail receptacle.

- Compartment Depth:  15 inches (381 mm).

- Cabinet:  0.10 inch (2.5 mm) thick welded aluminum sheet.

- Doors:  Minimum 0.125 inch (3 mm) thick aluminum, with stainless steel hardware and hinges.

- Locks:  5-pin cylinder cam lock with spring-loaded cover on each unit, 3 keys each lock. (USPS-1172 910 lock)

** NOTE TO SPECIFIER ** Delete pedestal if not required.

- Pedestal: Standard aluminum pedestal designed to ensure unit installed height meets all USPS installation requirements without additional adjustment.

  - 28.5 inch (724 mm) high pedestal standard with Types I, II and V.

  - 14.5 inch (368 mm) high pedestal standard with Types III, IV and VI.

** NOTE TO SPECIFIER ** Delete finish options not required in project.

- Finish:  Powder Coat Finish

  - Sandstone – standard

  - Black - standard

  - Dark Bronze - standard

  - White - standard

  - Forest Green - standard

  - Postal Gray - standard (for replacement units only)

** NOTE TO SPECIFIER ** Delete identification options not required in project.

- Compartment Identification:

  - Standard silver adhesive identification decals, 1.5 inch (38 mm) high by 1.75 inch (44 mm) wide, containing up to 5 characters

  - Optional engraved identifier, .75 inch (19 mm) high in 6 font selections up to 12 characters

  - Optional engraved identifier with black fill, .75 inch (19 mm) high in 6 font selections up to 12 characters

  - Optional engraved identifier with white fill, .75 inch (19 mm) high in 6 font selections up to 12 characters

** NOTE TO SPECIFIER ** Delete entire 2.3 below if not required.

2.3 	HIGH SECURITY CLUSTER BOX UNITS (HSCBU)

** NOTE TO SPECIFIER ** Florence Manufacturing is the ONLY manufacturer offering High Security CBU and therefore no equals exist.

High Security Cluster Box Units: Consisting of multiple compartments enclosed within freestanding, steel pedestal-mounted aluminum cabinet. Provide access to compartments for distributing incoming mail and collecting outgoing mail from front of unit by unlocking master lock and swinging pair of side-hinged master doors to provide accessibility to entire group of compartments. Provide access to each compartment for removing mail by swinging compartment door. Provide slot for outgoing mail collection with weather protection hood. Comply with USPS-B-1118 (“F” Specification) as manufactured exclusively by Florence Manufacturing.

** NOTE TO SPECIFIER ** Delete model type not required in project.

- Model:  vigilant™ series 1565 High Security Cluster Box Unit (HSCBU), the Florence vigilant™ 1565 High-Security Cluster Box Unit was designed specifically for the US Postal Inspection Service and is available for commercial use.

  - Model 1565-8, Type I: 8 compartments, 3.25 inches (83 mm) high, with 2 parcel compartments and 1 outgoing mail receptacle.

  - Model 1565-12, Type II: 12 compartments, 3.25 inches (83 mm) high, with 1 parcel compartment and 1 outgoing mail receptacle.

  - Model 1565-16, Type III: 16 compartments, 3.25 inches (83 mm) high, with 2 parcel compartments and 1 outgoing mail receptacle.

  - Model 1565-13, Type IV: 13 compartments, 5 inches (127 mm) high, with 1 parcel compartment and 1 outgoing mail receptacle.

- Compartment Depth:  15 inches (381 mm).

- Cabinet:  0.10 inch (2.5 mm) thick welded aluminum sheet.

- Doors:  Minimum 0.125 inch (3 mm) thick aluminum, with stainless steel hardware and hinges.

- Locks:  5-pin cylinder cam lock with spring-loaded cover on each unit, 3 keys each lock. (USPS-1172 910 lock)

** NOTE TO SPECIFIER ** Delete pedestal if not required.

- Pedestal: Standard steel pedestal designed to ensure unit installed height meets all USPS installation requirements without additional adjustment.

  - 28.5 inch (724 mm) high pedestal standard with Types I and II.

  - 14.5 inch (368 mm) high pedestal standard with Types III and IV.

** NOTE TO SPECIFIER ** Delete finish options not required in project.

- Finish:  Powder Coat Finish

  - Sandstone – standard

  - Black - standard

  - Dark Bronze - standard

  - White - standard

  - Forest Green - standard

  - Postal Gray - standard (for replacement units only)

** NOTE TO SPECIFIER ** Delete identification options not required in project.

- Compartment Identification:

  - Standard silver adhesive identification decals, 1.5 inch (38 mm) high by 1.75 inch (44 mm) wide, containing up to 5 characters

  - Optional engraved identifier, .75 inch (19 mm) high in 6 font selections up to 12 characters

  - Optional engraved identifier with black fill, .75 inch (19 mm) high in 6 font selections up to 12 characters

  - Optional engraved identifier with white fill, .75 inch (19 mm) high in 6 font selections up to 12 characters

** NOTE TO SPECIFIER ** Delete entire 2.4 below if not required.

2.4 	OUTDOOR PARCEL LOCKER (OPL)

** NOTE TO SPECIFIER ** Florence Manufacturing is the ONLY approved manufacturer of Outdoor Parcel Lockers and therefore no equals exist.

Outdoor Parcel Lockers: Consisting of multiple compartments enclosed within freestanding, pedestal-mounted aluminum cabinet. Provide access to compartments for distributing incoming parcels front of unit by unlocking master lock and swinging side-hinged master door(s) to provide accessibility to entire group of compartments. Provide access to each compartment for removing parcels by swinging compartment door with key-trapping lock. Comply with USPS-B-1118 (“F” Specification) as manufactured exclusively by Florence Manufacturing.

** NOTE TO SPECIFIER ** Delete model type not required in project.

- Model:  valiant™ series 1590 Outdoor Parcel Locker (OPL), the Florence valiant™ 1590 Outdoor Parcel Locker is designed to provide additional parcel lockers for Cluster Box Unit installations to meet USPS requirements; or to use alone for commercial parcel delivery.

  - Model 1590-T1, Type I: 2 parcel compartments, 19.75 inches (502 mm) high.

  - Model 1590-T2, Type II: 4 parcel compartments, 19.75 inches (502 mm) high.

- Compartment Depth:  15 inches (381 mm).

- Cabinet:  0.10 inch (2.5 mm) thick welded aluminum sheet.

- Doors:  Minimum 0.125 inch (3 mm) thick aluminum, with stainless steel hardware and hinges.

- Locks:  5-pin cylinder cam lock with spring-loaded cover on each unit, 3 keys each lock. (USPS-1172 910 lock)

** NOTE TO SPECIFIER ** Delete pedestal if not required.

- Pedestal: Standard aluminum pedestal designed to ensure unit installed height meets all USPS installation requirements without additional adjustment.

  - 14.5 inch (368 mm) high pedestal standard with Types I and II.

** NOTE TO SPECIFIER ** Delete finish options not required in project.

- Finish:  Powder Coat Finish

  - Sandstone – standard

  - Black - standard

  - Dark Bronze - standard

  - White - standard

  - Forest Green - standard

  - Postal Gray - standard (for replacement units only)

** NOTE TO SPECIFIER ** Delete identification options not required in project.

- Compartment Identification:

  - Standard silver adhesive identification decals, 1.5 inch (38 mm) high by 1.75 inch (44 mm) wide, containing up to 5 characters

  - Optional engraved identifier, .75 inch (19 mm) high in 6 font selections up to 12 characters

  - Optional engraved identifier with black fill, .75 inch (19 mm) high in 6 font selections up to 12 characters

  - Optional engraved identifier with white fill, .75 inch (19 mm) high in 6 font selections up to 12 characters

** NOTE TO SPECIFIER ** Delete entire 2.5 below if not required.

2.5 	DECORATIVE ACCESSORIES

Decorative Accessories: Consisting of cap and pedestal covers for mounting on cluster box units. Comply with USPS-B-1118 (“F” Specification) as manufactured by Florence Manufacturing.

** NOTE TO SPECIFIER ** Delete model type not required in project. All classic options are exclusive to Florence Manufacturing and therefore no equals exist.

- Series:  DESIGNER CAP

  - Model VOGUEC1, Classic Crown Cap for all CBU Types

  - Model VOGUECA, Traditional Finial Cap for all CBU Types

- Series: PEDESTAL COVER

  - Model VOGUEP128, Classic Pillar Pedestal Cover for CBU Type I, II, V

  - Model VOGUEPA, Traditional Column Pedestal Cover for CBU Type I, II, V

  - Model VOGUEP114, Classic Pillar Pedestal Cover for CBU Type III, IV, VI

  - Model VOGUEPA14, Traditional Column Pedestal Cover for CBU Type III, IV, VI

** NOTE TO SPECIFIER ** Delete finish options not required in project.

- Finish:  Powder Coat Finish

  - Sandstone – standard

  - Black - standard

  - Dark Bronze - standard

  - White - standard

  - Forest Green - standard

  - Postal Gray - standard (for replacement units only)

- EXECUTION

  - EXAMINATION

    - Installer's Examination:

- Examine conditions under which construction activities of this section are to be performed; submit written notification if such conditions are unacceptable.

- Verify that cast concrete pads are correctly located, aligned and sized for mailboxes.

- Beginning installation indicates acceptance of conditions.

  - PREPARATION

    - Clean surfaces thoroughly prior to installation.

  - INSTALLATION

    - Install mailboxes in accordance with shop drawings and manufacturer's printed installation instructions.

    - Align, plumb, and level; anchor in accordance with manufacturer's requirements.

  - ADJUSTING

    - Adjust doors and locks to operate correctly.

  - CLEANING

    - Clean surfaces with mild dish detergent.  Do not use harsh abrasive cleaners.  Lubricate locks with graphite type lubricants only. Refer to manufacturer’s recommended methods.

  - PROTECTION OF INSTALLED PRODUCTS

    - Protect finishes from damage by construction activities. Touch-up, repair or replace damaged products before Substantial Completion.

END OF SECTION